%Appendix 1

Calculation of power distribution.

%%%%% talking about how to calculate the power in each section base on FIT knowledge. In %Appendix_power_distribution

This section aims for offering a set of numerical functions to calculating the power distribution along beam propagation in a photonic waveguide. The simplest idea is to compute the power in waveguide and base by integrating power flow density (or Poynting vector $\vec{S}$) at their cross-sections respectively. Meanwhile CST MWS has constructed a cuboid like Fig.\quad\ref{Afig:app_power_distribution01}, which contains all engaged objects such as waveguide and TLF, as a total calculation space which is discretized through Finite Integral method (FIT) and all variables(see \cite{script_FeldSim} or section Finite Integration Method) from FIT are also available from CST MWS. In following it will be introduced how to calculate the power distribution of a Fiber-to-Chip model (see chapter modelling).

\begin{figure}[ht]

\centering

\includegraphics[width=0.5 \textwidth]{bilder/basic_waveguide_efield}

\caption{Calculation Cuboid in CST}

\label{Afig:app_power_distribution01}

\end{figure}

The first step is to choose a working plane. Suppose a working plane through point z$_{0}$ at z axis in the total calculation space Fig.\quad\ref{Afig:app_power_distribution01} is given, the point index n$_{0}$ can be determined by its coordinate z$_{0}$. The plane Fig. \quad\ref{Afig:app_power_distribution02}  is divided into small elemental pieces in FIT. The base cross-section is given by four points coordinates (x$_{1}$, y$_{1}$),(x$_{2}$, y$_{2}$),(x$_{3}$, y$_{3}$) and (x$_{4}$, y$_{4}$). Their points indexes (n$_{1}$, n$_{2}$, n$_{3}$, n$_{4}$) can also be derived.

\begin{figure}[ht]

\centering

\includegraphics[width=0.5 \textwidth]{bilder/basic_waveguide_efield}

\caption{Cross-section at z$_{0}$}

\label{Afig:app_power_distribution02}

\end{figure}

In the next step it is necessary to prepare variables such as elemental

As is in \ref{script_FeldSim } the complete elemental plane matrix D$_{A}$ is given by (\ref{Aeq:da_matrix}):

\begin{equation}

D_{A}=Diag\left\{Ax(1),\cdots,Ax(Np),Ay(1),\cdots,Ay(Np), Az(1),\cdots,Az(Np)\right\}

\label{Aeq:da_matrix}

\end{equation}

In the working cross-section only z-components of elemental plane matrix D$_{A}$ are needed:

\begin{equation}

D_{Az}=D_{A}(2*Np+1:3*Np, 2*Np+1:3*Np)

\label{Aeq:daz_matrix}

\end{equation}

Construct a auxiliary matrix $A_{base}$, which composed of only $1$ and $0$ like Fig\quad\ref{ Afig:app_Auxiliary_matrix}, to indicate all points indexes which are included in base cross-section.

\begin{equation}

A_{base}=Diag\left\{0,\cdots 0,P1,0,\cdots 0,P2,0,\cdots 0\right\}

\label{Aeq:A_matrix}

\end{equation}

\begin{figure}[ht]

\centering

\includegraphics[width=0.5\textwidth]{bilder/basic_waveguide_efield}

\caption{structure of the auxiliary matrix $A$}

\label{Afig:app_Auxiliary_matrix}

\end{figure}

Where P$_{x}$ are submateix:

\begin{equation}

P_{x}=Diag\left\{1,\cdot,1\right\}_{(n_{2}-n_{1})*(n_{2}-n_{1})}

\end{equation}

And m is given by:

\begin{equation}

M=(n_{3}-n_{1})/n_{x}=(n_{4}-n_{2})/n_{x}

\end{equation}

Then

\begin{equation}

D_{Abase}=A_{guide}*D_{A}

\end{equation}

Pick z-components of the Poynting vector $S$:

\begin{equation}

S_{z}=S(2*Np+1:3*Np)

\end{equation}

At last the power in base at the plane (for z=z$_{0}$) can be counted up by:

\begin{equation}

P_{base}(z)=sum(D_{Aguide}*S_{z})

\end{equation}

By analogous procedures power in guide cross-section$P_{guide}$ and in total cross-section $P_{total}$ are derived. For observing the power distribution the results are processed by normalization:

\begin{align}

\eta_{guide}&=\frac{P_{guide}}{P_{total}}\\

\eta_{base}&=\frac{P_{base}}{P_{total}}\\

\eta_{air}&=1-\eta_{guide}-\eta_{base}

\end{align}